Data Logging of Temperature and Humidity Values Using ESP32 with RS485 and RTC into SD Card

Code:-

#include <Wire.h>

#include <ModbusMaster.h>

#include <SPI.h>

#include <SD.h>

#include <RTClib.h>

// RS485 (XY-MOD02) Connections

#define RS485_TX_PIN 33

#define RS485_RX_PIN 32

#define RS485_ENABLE_PIN 4

// SD Card Module (SPI) Connections

#define SD_CS_PIN 5  // Chip Select for SD Card

ModbusMaster node;

RTC_DS3231 rtc;

File logFile;

// Pre-transmission function

void preTransmission() {

digitalWrite(RS485_ENABLE_PIN, HIGH);

}

// Post-transmission function

void postTransmission() {

digitalWrite(RS485_ENABLE_PIN, LOW);

}

void setup() {

Serial.begin(115200);

// Initialize RS-485 communication

pinMode(RS485_ENABLE_PIN, OUTPUT);

digitalWrite(RS485_ENABLE_PIN, LOW);

Serial1.begin(9600, SERIAL_8N1, RS485_RX_PIN, RS485_TX_PIN);

node.begin(1, Serial1);

node.preTransmission(preTransmission);

node.postTransmission(postTransmission);

// Initialize RTC

Wire.begin();

if (!rtc.begin()) {

Serial.println("RTC not found!");

while (1);

}

if (rtc.lostPower()) {

Serial.println("RTC lost power, setting time...");

rtc.adjust(DateTime(__DATE__, __TIME__)); // Set RTC to compile time

}

// OPTIONAL: Manually set RTC time (use once, then remove or comment out)

// rtc.adjust(DateTime(2025, 1, 29, 14, 30, 0)); // YYYY, MM, DD, HH, MM, SS

// Initialize SD Card

if (!SD.begin(SD_CS_PIN)) {

Serial.println("SD Card Initialization Failed!");

} else {

Serial.println("SD Card Ready.");

}

// Create the log file if it doesn't exist

if (!SD.exists("/data_log.txt")) {

logFile = SD.open("/data_log.txt", FILE_WRITE);

if (logFile) {

logFile.println("Timestamp, Temperature (°C), Humidity (%)");

logFile.close();

}

}

}

void loop() {

// 1️⃣ Read Current Time from RTC

DateTime now = rtc.now();

String timestamp = String(now.day()) + "-" + String(now.month()) + "-" + String(now.year()) + " " +

String(now.hour()) + ":" + String(now.minute()) + ":" + String(now.second());

// 2️⃣ Read Temperature & Humidity from XY-MOD02

uint8_t result;

uint16_t tempData, humidityData;

float temperature = -999, humidity = -999;

result = node.readInputRegisters(0x0001, 1);

if (result == node.ku8MBSuccess) {

tempData = node.getResponseBuffer(0);

temperature = tempData / 10.0;

} else {

Serial.println("Error reading temperature.");

}

result = node.readInputRegisters(0x0002, 1);

if (result == node.ku8MBSuccess) {

humidityData = node.getResponseBuffer(0);

humidity = humidityData / 10.0;

} else {

Serial.println("Error reading humidity.");

}

// 3️⃣ Format Data String

String dataString = timestamp + ", " + String(temperature, 1) + "°C, " + String(humidity, 1) + "%";

// 4️⃣ Send Data Over RS485

Serial.println("Sending via RS485: " + dataString);

Serial1.println(dataString); // Send as ASCII over RS485

// 5️⃣ Store Data in SD Card

logFile = SD.open("/data_log.txt", FILE_APPEND);

if (logFile) {

logFile.println(dataString);

logFile.close();

} else {

Serial.println("Failed to write to SD card!");

}

delay(2000);

}